ĐẠI HỌC HOA SEN KHOA CÔNG NGHỆ THÔNG TIN ---- ---- BÁO CÁO PROJECT Tên sinh viên: Lê Minh Quang Mssv: 22122997 Tên đề tài: Cảm biến độ ẩm đất Môn học: Thực hành công nghệ thông tin 3 Lớp: 0200 Giảng viên hướng dẫn: Trần Thị Trương Thi THÀNH PHỐ HỒ CHÍ MINH – 2024

NỘI DUNG

Thiết bị

Những thiết bị sử dụng:

- Wemos D1 R2, cảm biến độ ẩm đất, máy bơm nước mini, Relay 5v, dây nối, breadboard MB-102 830 lỗ.

\

Code Arduino

#include <ESP8266WiFi.h>

#include <PubSubClient.h>

#include <ArduinoJson.h>

#include <NTPClient.h>

#include <WiFiUdp.h>

#include <ESP8266HTTPClient.h>  // Include the HTTPClient library

// WiFi credentials

// #define ssid "HSU_Students"

// #define password "dhhs12cnvch"

#define ssid "Tenda_CBCCD0"

#define password "t12345678"

// MQTT broker details

#define mqtt_server "broker.emqx.io"

const uint16_t mqtt_port = 1883;

#define mqtt_topic_pub_led "ttcntt3/mqleeee/ledstatus"

#define mqtt_topic_sub_led "ttcntt3/mqleeee/ledstatus"

#define mqtt_topic_pub_soil "ttcntt3/mqleeee/soil"

#define mqtt_topic_sub_soil "ttcntt3/mqleeee/soil"

#define mqtt_topic_pub_time "ttcntt3/mqleeee/time"

#define mqtt_topic_sub_time "ttcntt3/mqleeee/time"

// Sensor and LED pin definitions

#define soil A0

#define LED_PIN D4

WiFiClient espClient;

PubSubClient client(espClient);

StaticJsonDocument<256> doc;

WiFiUDP ntpUDP;

NTPClient timeClient(ntpUDP, "pool.ntp.org", 7 * 3600, 60000);  // UTC+7, update every 60 seconds

const char* URL = "http://192.168.1.87:3003/data";  // Your server URL

char ledstatus[32] = "off";

void setup() {

pinMode(soil, INPUT);

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);  // Set initial state to OFF

Serial.begin(115200);

setup_wifi();

client.setServer(mqtt_server, mqtt_port);

client.setCallback(callback);

timeClient.begin();

reconnect();

}

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

}

void callback(char* topic, byte* payload, unsigned int length) {

deserializeJson(doc, payload, length);

if (String(topic) == mqtt_topic_sub_led) {

strlcpy(ledstatus, doc["status"] | "off", sizeof(ledstatus));

String mystring(ledstatus);

Serial.print("Message arrived [");

Serial.print(topic);

Serial.print("] ");

Serial.println(ledstatus);

if (mystring == "on") {

Serial.println("Turning LED on");

digitalWrite(LED_PIN, HIGH);

} else {

Serial.println("Turning LED off");

digitalWrite(LED_PIN, LOW);

}

}

}

void reconnect() {

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

if (client.connect("ttcntt3_mqleeee")) {

Serial.println("connected");

// Xuất bản trạng thái LED ban đầu

doc.clear();

doc["name"] = "led";

doc["status"] = ledstatus;

char buffer[256];

size_t n = serializeJson(doc, buffer);

client.publish(mqtt_topic_pub_led, buffer, n);

// Xuất bản thời gian ban đầu

doc.clear();

doc["timestamp"] = timeClient.getFormattedTime();

n = serializeJson(doc, buffer);

client.publish(mqtt_topic_pub_time, buffer, n);

// Xuất bản độ ẩm đất ban đầu

doc.clear();

doc["soilMoisture"] = analogRead(soil);

n = serializeJson(doc, buffer);

client.publish(mqtt_topic_pub_soil, buffer, n);

client.subscribe(mqtt_topic_sub_led);

client.subscribe(mqtt_topic_sub_soil);

client.subscribe(mqtt_topic_sub_time);

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void postData(int soilMoistureValue, const String& timestamp) {

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(espClient,URL);

http.addHeader("Content-Type", "application/json");

StaticJsonDocument<256> jsonDoc;

jsonDoc["soilMoisture"] = soilMoistureValue;

jsonDoc["timestamp"] = timestamp;

String requestBody;

serializeJson(jsonDoc, requestBody);

int httpResponseCode = http.POST(requestBody);

if (httpResponseCode > 0) {

String response = http.getString();

Serial.println(httpResponseCode);

Serial.println(response);

} else {

Serial.print("Error on sending POST: ");

Serial.println(httpResponseCode);

}

http.end();

} else {

Serial.println("WiFi Disconnected");

}

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop();

static unsigned long lastPublish = 0;

timeClient.update();

unsigned long now = millis();

if (now - lastPublish >= 15000) {  // Corrected to use >= instead of ==

// Publish soil moisture periodically

int soilMoistureValue = analogRead(soil);

String formattedTime = timeClient.getFormattedTime();

doc.clear();

doc["soilMoisture"] = soilMoistureValue;

char buffer[256];

size_t n = serializeJson(doc, buffer);

client.publish(mqtt_topic_pub_soil, buffer, n);

Serial.print("Soil Moisture: ");

Serial.print(soilMoistureValue);

Serial.print(" at ");

Serial.println(formattedTime);

doc.clear();

doc["timestamp"] = formattedTime;

n = serializeJson(doc, buffer);

client.publish(mqtt_topic_pub_time, buffer, n);

lastPublish = now;

// Post data to the server

postData(soilMoistureValue, formattedTime);

}

}

MQTTX

Front-end

Back-end

const express = require('express');

const mysql = require('mysql2');

const cors = require('cors');

const app = express();

const port = 3002;

const bodyParser = require('body-parser');

// Middleware

app.use(cors());

app.use(express.json()); // Body parser middleware

// Thiết lập kết nối cơ sở dữ liệu MySQL

const db = mysql.createConnection({

host: 'db',

user: 'root',

password: 'Quangdeptrai123', // Thay đổi mật khẩu tương ứng

database: 'mydatabase'

});

app.use(bodyParser.json());  // Parse JSON bodies

// Kết nối đến MySQL

db.connect((err) => {

if (err) {

throw err;

}

console.log('MySQL connected...');

});

// Endpoint: Lấy tất cả các ngày trong tuần

app.get('/time_schedule', (req, res) => {

const sql = 'SELECT * FROM time_schedule ORDER BY id DESC LIMIT 1';

db.query(sql, (err, results) => {

if (err) {

console.error(err);

res.status(500).json({ error: 'Error fetching data' });

return;

}

res.json(results);

});

});

app.post('/time_schedules', (req, res) => {

const { timestart } = req.body;

const sql = 'INSERT INTO time_schedule (timestart) VALUES (?)';

db.query(sql, [timestart], (err, result) => {

if (err) throw err;

res.send('Time schedule saved');

});

});

app.post('/data', (req, res) => {

const { soilMoisture, timestamp } = req.body;

const sql = 'INSERT INTO data (soilMoisture, timestamp) VALUES (?, ?)';

db.query(sql, [soilMoisture, timestamp], (err, result) => {

if (err) {

console.error('Error inserting data:', err);

res.status(500).send('Error saving data');

return;

}

res.send('Data saved');

});

});

app.listen(port, () => {

console.log(`Server running on port ${port}`);

});

Docker

6.1 dockerfile

FROM node:lts-alpine

ENV NODE_ENV=production

WORKDIR /usr/src/app

COPY ["package.json", "package-lock.json*", "npm-shrinkwrap.json*", "./"]

RUN npm install --production --silent && mv node_modules ../

COPY . .

EXPOSE 5555

RUN chown -R node /usr/src/app

USER node

CMD [ "node", "api.js" ]

6.2 docker-compose.yml

version: '3.4'

services:

db:

image: mysql

container_name: mysql_db

environment:

MYSQL_ROOT_PASSWORD: Quangdeptrai123

MYSQL_DATABASE: mydatabase

ports:

- "3308:3306"

volumes:

- mysql_db-data:/var/lib/mysql

networks:

- network-mysql-api

backend:

depends_on:

- db

networks:

- network-mysql-api

image: backend

container_name: cntt3-api

build:

dockerfile: ./Dockerfile

environment:

NODE_ENV: production

DB_HOST: db

DB_USER: root

DB_PASSWORD: Quangdeptrai123

DB_NAME: mydatabase

DB_PORT: 3306

ports:

- "3003:3002"

volumes:

mysql_db-data:

networks:

network-mysql-api:

API